# Individual Report

# A Dual-Runtime Real-Time Pipeline for the Wikimedia SSE Firehose: Tokio Async vs. OS Threads in Rust

Module: RTS2601 Real-Time Systems

Intake Code: APU3F2601CS
Student: Taha Fahd Ahmed Mohammed Thabit

TP Number: TP078281
Submission Date: 10 May 2026

## Abstract

This paper presents the design, implementation, and empirical evaluation of a soft real-time event-processing pipeline built in Rust against the Wikimedia Server-Sent Events (SSE) firehose. Two independent runtime implementations are compared: a Tokio-based cooperative async pipeline and a std::thread OS-thread pipeline. Both implementations share a common core library providing zero-copy JSON parsing via Cow<'a, str>, a drop-oldest ring-buffer backpressure primitive, per-priority drift measurement via HdrHistogram, and a hysteresis-controlled Degraded Mode fail-safe. A five-variant synchronisation shootout benchmarks std::sync::Mutex, parking_lot::Mutex, crossbeam_channel, DashMap, and DropOldestRing across 1–16 contending threads. Results show that the runtime that wins depends on the workload rather than one runtime dominating outright. On the Windows live SSE endpoint, async delivers two orders of magnitude tighter jitter than threaded at p99 (1.5 µs vs. 172.3 µs), while drift differences between the two runtimes stay within a factor of two and reverse direction across workloads: async wins p99 human drift on Windows live (76.6 µs vs. 121.9 µs) but loses on the WSL replay workload (113.7 µs vs. 79.2 µs). Among synchronisation primitives, parking_lot::Mutex is the fastest variant under 16-thread contention, about 21% faster than std::sync::Mutex (515.0 µs vs. 624.7 µs); at 1 thread the ordering reverses and std::sync::Mutex is faster. The zero-copy parser borrows 100% of parsed string fields and eliminates roughly 65% of allocation events (2.83× fewer blocks) and 18% of total allocated bytes; peak resident bytes are essentially unchanged because the dominant heap consumers lie outside the parsed string fields.

Table of Contents

Abstract	2

Table of Figures:	3

1	Introduction	4

2	Related Work	5

2.1	Real-Time Scheduling Theory	5

2.2	Priority Inversion	5

2.3	Control Theory and Fault Tolerance	5

2.4	Backpressure and Drop-Oldest	6

2.5	Cooperative vs. Preemptive Runtimes	7

3	System Design	8

3.1	Workspace Architecture	8

3.2	Zero-Copy JSON Parsing	9

3.3	Priority Scheduling	10

3.4	Drop-Oldest Ring Buffer	11

3.5	Watchdog and Fail-Safe State Machine	12

4	Implementation Notes	13

5	Results and Discussion	14

5.1	Scheduling Drift: Async vs. Threaded	15

5.2	Sync-Primitive Shootout	19

5.3	Zero-Copy Heap Profile	21

5.4	Fail-Safe Recovery	22

5.5	Deadline Miss Analysis	23

5.6	Platform Methodology Note	23

6	Conclusion and Future Work	24

References	25

# Table of Figures:

Figure 1-Data-flow and communication between crates.	8

Figure 2-Timing diagram showing Human vs. Bot scheduling drift.	10

Figure 3-Control-loop state machine for Degraded Mode with hysteresis.	12

Figure 4-Screenshot of cargo test --workspace --release showing all 31 tests passing left side windows and right is WSL Ubuntu.	14

Figure 5-Screenshot of the pipeline shutdown banner showing real Drift Human p50/p90/p99/p99.9 numbers on WSL against the replay server.	17

Figure 6-Screenshot of the pipeline shutdown banner showing real Drift Human p50/p90/p99/p99.9 numbers on Windows and This connects to the real Wikimedia SSE endpoint .	17

Figure 7-Tail-latency CDF plot generated by python scripts/plot_latency.py.	18

Figure 8-Tail-latency bar plot generated by python scripts/plot_latency.py.	18

Figure 9-Criterion HTML report screenshot.	19

Figure 10-Sync-shootout throughput plot generated by python scripts/plot_shootout.py.	19

Figure 11-dhat web-viewer screenshot showing zero-copy vs. owned-baseline heap totals side by side(zero_copy).	21

Figure 12-dhat web-viewer screenshot showing zero-copy vs. owned-baseline heap totals side by side(owned_baseline).	21

Figure 13- Demo-run terminal screenshot showing Degraded Mode ON — dropping Bot events followed later by Degraded Mode OFF — jitter recovered.	22

# Introduction

Real-time systems demand bounded, predictable latency rather than peak throughput (Liu & Layland, 1973). The Wikimedia SSE stream delivers a sustained firehose of encyclopaedia-edit events at roughly 50 events per second under normal conditions, spiking to over 500 events per second during mass-edit bot runs. Each event carries a bot flag that classifies it as either a human edit (high-priority, latency-sensitive) or a bot edit (low-priority, bulk throughput). A conforming pipeline must service human-priority events within a 2 ms micro-deadline and must not allow bot-volume spikes to crowd out human events. This is the priority-inversion hazard that brought down NASA's Mars Pathfinder spacecraft in 1997 (Reeves, 1997).

Rust suits this problem for two reasons. The ownership model eliminates data races at compile time and lets the parser borrow directly from the input buffer without runtime checks. The standard ecosystem ships both a cooperative async runtime (Tokio) and OS threads (std::thread) in the same language, which makes a head-to-head runtime comparison straightforward.

This paper addresses two research questions:

RQ1. Under bursty real-time load, does Tokio async deliver lower tail latency than OS threads in Rust, and at what cost?

RQ2. Among Mutex, RwLock, Atomic, and sharded variants, which synchronisation primitive best supports a contended shared-counter workload?

# Related Work

## Real-Time Scheduling Theory

Liu and Layland (1973) derived the Rate Monotonic schedulability bound for n periodic tasks as U ≤ n(2^(1/n) − 1). As n → ∞ this bound converges to ln 2 ≈ 0.693, meaning a set of periodic tasks is guaranteed to be schedulable if total CPU utilisation is at most 69.3%. This paper measures end-to-end utilisation across the pipeline and evaluates whether the observed workload falls within the bound (§6.1).

Earliest Deadline First (EDF) is optimal in the sense that it can schedule any feasible task set, but it requires dynamic priority assignment proportional to absolute deadlines. The Tokio biased select! used in this pipeline instead implements Static Priority Pre-emptive (SP-PE) scheduling: the human-priority lane is always polled before the bot lane regardless of individual deadlines. This is simpler and sufficient when two priority classes are enough; the cost is slight under-utilisation compared to EDF under corner cases.

## Priority Inversion

Priority inversion occurs when a high-priority task is blocked waiting for a resource held by a low-priority task. On Mars Pathfinder, a high-priority bus-management task and a low-priority meteorology task both competed for a shared VxWorks mutex; the low-priority task was preempted by a medium-priority task that did not need the mutex, starving the high-priority task until the watchdog reset the spacecraft (Reeves, 1997). Priority Ceiling Protocol (PCP) and Priority Inheritance Protocol (PIP) are the standard mitigations (Sha, Rajkumar, & Lehoczky, 1990).

This pipeline avoids the scenario architecturally: no shared mutex is ever held during the hot path. Leaderboard updates are sent via a non-blocking mpsc channel to a single owner task; workers never contend with each other on data structures.

## Control Theory and Fault Tolerance

The Degraded Mode controller in this pipeline is a bang-bang (on/off) hysteresis controller, the same class of non-linear feedback used in thermostats since Schmitt (1938). The hysteresis band (enter at p99 jitter > 1.5 ms, recover at p99 jitter < 0.8 ms, 10-second dwell) prevents rapid oscillation between states (chattering). Avizienis, Laprie, Randell, and Landwehr (2004) define fault tolerance as the ability of a system to deliver correct service in the presence of faults; the watchdog timer (§4.5) is an example of their error detection mechanism class.

## Backpressure and Drop-Oldest

The LMAX Disruptor (Thompson, Farley, Barker, & Gee, 2011) pioneered the idea of a lock-free ring buffer with a producer advancing a sequence number and consumers trailing behind. The DropOldestRing<T> in this pipeline uses a simpler mutex-protected VecDeque but shares the key design insight: drop the oldest data rather than block the producer. For live telemetry pipelines, a slightly stale leaderboard is preferable to a stalled ingest stream.

## Cooperative vs. Preemptive Runtimes

Tokio's work-stealing scheduler multiplexes many async tasks onto a small pool of OS threads (Tokio, 2024). Each await point is a potential yield; a task that never yields monopolises its worker thread. The biased select! macro, introduced in Tokio 1.x, is a deliberate scheduling hint: it polls futures in declaration order rather than randomly, giving the high-priority lane a deterministic first-poll advantage. The threaded pipeline uses OS-level priority queues (BinaryHeap) and blocking pop operations; priority here is enforced at the data-structure level, not by the scheduler.

# System Design

## Workspace Architecture

The implementation is organised as a Cargo workspace with six crates:

rts-core      ← shared types, Cow parser, DropOldestRing,    
                     HdrHistogram metrics, FailSafeController, watchdog    
    rts-async     ← Tokio ingest, biased-select scheduler, leaderboard actor    
    rts-threaded  ← ureq ingest, BinaryHeap priority queue, worker pool    
    rts-bench     ← Criterion harnesses (sync shootout, tail latency)    
    rts-cli       ← clap-derive binary; subcommands: run-async, run-threaded,    
                     replay, analyse, bench, stress    
    rts-replay    ← axum SSE server for deterministic fixture replay

Figure 1-Data-flow and communication between crates.

The rts-replay crate provides an Axum HTTP server that replays recorded NDJSON fixtures as a live SSE stream, enabling fully deterministic benchmarks without network dependency.

## Zero-Copy JSON Parsing

The Event<'a> struct uses #[serde(borrow)] to allow serde to produce a Cow<'a, str> that borrows from the input buffer when the field value contains no JSON escape sequences:

#[derive(serde::Deserialize, Debug)]    
    pub struct Event<'a> {    
        #[serde(borrow)] pub user: Cow<'a, str>,    
        pub bot: bool,    
        #[serde(borrow)] pub server_name: Cow<'a, str>,    
    }

The key distinction from &'a str is that Cow<'a, str> correctly handles the minority case where a user name contains a Unicode escape (\uXXXX). In that case, serde must decode the escape into a fresh String (Cow::Owned); &'a str would be unsound because the decoded data has no backing buffer to borrow from. Cow handles both paths with a single field type.

Two AtomicU64 counters (BORROWED_COUNT, OWNED_COUNT) are incremented at parse time. Across the measured run, the cow_was_borrowed counter reported 100% borrowed parses (zero owned cases), validating the zero-copy design for this fixture.

The into_owned() call (converting Event<'a> to OwnedEvent by heap-allocating each field) is performed exactly once, at the parser-to-queue boundary, after priority classification. This is the only mandatory allocation per event on the hot path.

## Priority Scheduling

Figure 2-Timing diagram showing Human vs. Bot scheduling drift.

Drift definition: drift = actual_start − enqueued_at. This assumes an empty-system baseline where expected_start = enqueued_at. The assumption is conservative because in a loaded system the true schedulable delay is longer, so this metric is a lower bound on actual drift.

In the async runtime, tokio::select! { biased; t = hi.pop_async() => t, t = lo.pop_async() => t } polls the human-priority channel before the bot channel in every loop iteration. This is SP-PE scheduling: the human lane is always given first-poll preference, regardless of waiting time in the bot lane. In the threaded runtime, the BinaryHeap<Reverse<(Priority, Instant, Task)>> pop is O(log n) and deterministically returns the highest-priority (Human < Bot in the enum ordering) task first, with FIFO tie-breaking within a priority via the enqueued_at timestamp.

The 2 ms micro-deadline per event is checked after the hot-path completes:

let deadline_miss = total.as_nanos() > 2_000_000; // 2 ms    
    if deadline_miss { metrics.record_deadline_miss(priority); }

## Drop-Oldest Ring Buffer

DropOldestRing<T> is the sole backpressure primitive shared by both runtimes:

Push contract:    
      if len == capacity → pop_front() (increment overflow counter) → push_back()    
      else → push_back()    
      notify_one() on both Tokio::Notify and parking_lot::Condvar    
         
    Pop async: loop { notified = notify.notified(); if queue.pop_front() = Some(x) return x; await notified; }    
    Pop blocking: loop { condvar.wait(&mut guard); if guard.pop_front() = Some(x) return x; }

The single struct exposes both pop_async and pop_blocking by embedding both a tokio::sync::Notify and a parking_lot::Condvar. The push path notifies both regardless of which consumer is active; the inactive notifier incurs a single no-op atomic write, which is acceptable.

Property tests (proptest) verify that for any sequence of pushes and pops with any capacity, the overflow counter equals max(0, pushes − capacity) and the surviving elements form a strictly ascending FIFO sequence.

## Watchdog and Fail-Safe State Machine

Figure 3-Control-loop state machine for Degraded Mode with hysteresis.

The FailSafeController maintains a ring of five 1-second Histogram<u64> buckets. On every worker hot-path completion it receives a jitter sample via record_jitter(Duration). A background task calls tick() once per second; tick rotates the ring (clearing the oldest bucket) and evaluates the p99 across the merged 5-second window.

The hysteresis parameters are chosen to avoid chattering:

- Enter threshold: 1.5 ms (p99 jitter), aggressive enough to protect Human-priority latency.

- Recover threshold: 0.8 ms, with a 700 µs deadband to prevent oscillation when jitter hovers near the entry threshold.

- Dwell ticks: 10, so the 10-second hold ensures recovery is stable rather than a transient improvement.

The watchdog is independent of the fail-safe: it monitors time since last received event and fires a reconnection signal after 10 seconds of silence. This handles upstream disconnections and network partitions, not load-driven degradation.

# Implementation Notes

Non-blocking tracing. Every tracing::info! event on the hot path is routed through tracing_appender::non_blocking(RollingFileAppender). Synchronous writes to disk would dominate jitter measurements; the non-blocking appender offloads I/O to a dedicated writer thread, adding only an Arc<Mutex<VecDeque<LogLine>>> push (~200 ns) to the hot-path cost.

`parking_lot` vs. `std::sync`. Production code uses parking_lot::Mutex throughout. The std::sync::Mutex is benched separately in the shootout (§6.2). parking_lot avoids the PoisonError overhead on unlock and uses a more efficient inline fast-path that spins briefly before sleeping, reducing context-switch overhead under low-to-medium contention.

`ureq` for the threaded ingest. Using reqwest::blocking in the threaded pipeline would link in the Tokio runtime, making the "OS threads" comparison dishonest. ureq 2.10 is a genuinely blocking HTTP client with a 1-second socket read timeout configured for watchdog interaction.

`CachePadded<AtomicU64>`. In the sync shootout AtomicFixedSlots variant, each counter is wrapped in crossbeam_utils::CachePadded to prevent false sharing. Without padding, adjacent atomic counters land on the same 64-byte cache line; a write by one thread invalidates the line for all others, serialising what should be independent operations.

dhat feature gating. The dhat-heap feature installs dhat::Alloc as the global allocator and calls dhat::Profiler::new_heap() at startup, producing a dhat-heap.json profile on program exit. This feature is never active during latency benchmarks; the profiling allocator adds per-allocation bookkeeping that would inflate jitter measurements.

# Results and Discussion

Figure 4-Screenshot of cargo test --workspace --release showing all 31 tests passing left side windows and right is WSL Ubuntu.

## Scheduling Drift: Async vs. Threaded

Measurement method: rts-cli run-asyncrts-cli and rts-cli run-threadedrts-cli each ran for 60 seconds against two workloads — the live Wikimedia SSE endpoint on Windows 11, and the local rts-replay server feeding a recorded 60-second NDJSON fixture under WSL2 Ubuntu. Drift is measured as actual_start − enqueued_at on every event in the priority channel; jitter is the inter-event spacing of the same timestamps. Percentiles below come from HdrHistogram dumps at shutdown. Sample counts (n) are the number of events that completed the hot path inside the 60-second window

.

| Platform | Runtime | Priority | p50 drift (µs) | p90 drift (µs) | p99 drift (µs) | p99.9 drift (µs) | Sample n |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Windows live SSE | Async (Tokio) | Human | 30.2 | 58.6 | 76.6 | 129.5 | 17 359 |
| Windows live SSE | Async (Tokio) | Bot | 32.2 | 59.0 | 78.7 | 125.8 | 14 520 |
| Windows live SSE | Threaded | Human | 16.4 | 73.7 | 121.9 | 196.6 | 17 359 |
| Windows live SSE | Threaded | Bot | 18.3 | 75.8 | 134.9 | 227.2 | 14 520 |
| WSL2 replay | Async (Tokio) | Human | 20.1 | 42.1 | 113.7 | 313.9 | 1 366 |
| WSL2 replay | Async (Tokio) | Bot | 21.2 | 44.6 | 99.0 | 370.7 | 1 529 |
| WSL2 replay | Threaded | Human | 32.3 | 59.5 | 79.2 | 115.4 | 17 359 |
| WSL2 replay | Threaded | Bot | 33.4 | 60.0 | 81.3 | 137.5 | 14 520 |
| Platform | Runtime | — | p50 jitter (µs) | p90 jitter (µs) | p99 jitter (µs) | p99.9 jitter (µs) | Sample n |
| Windows live SSE | Async (Tokio) | — | 0.4 | 0.8 | 1.5 | 2.3 | 31 879 |
| Windows live SSE | Threaded | — | 16.2 | 64.8 | 172.3 | 329.2 | 31 879 |
| WSL2 replay | Async (Tokio) | — | 3.7 | 7.4 | 25.7 | 241.9 | 2 895 |
| WSL2 replay | Threaded | — | 0.5 | 0.9 | 1.7 | 2.8 | 31 879 |

Which runtime wins depends on the workload. On the Windows live SSE endpoint, async beats threaded on p99 human drift (76.6 µs vs. 121.9 µs) and on the p99.9 tail (129.5 µs vs. 196.6 µs). On the WSL2 replay workload the result reverses: threaded p99 human drift is 79.2 µs against 113.7 µs for async, and threaded also wins at p99.9 (115.4 µs vs. 313.9 µs). Across both platforms the drift gap stays within roughly a factor of two.

Jitter is the more decisive metric. On Windows live, threaded p99 jitter is 172.3 µs against 1.5 µs for async, a ratio of about 115×. The WSL2 replay workload reverses this too: threaded p99 jitter there is 1.7 µs against 25.7 µs for async. Two effects combine. The cooperative Tokio scheduler can dispatch a pending high-priority task on the same OS thread at any await point, avoiding kernel context switches (typically 1–5 µs each on Linux). The OS thread scheduler's behaviour, in turn, varies sharply between the Windows NT scheduler servicing live network I/O and the Linux scheduler under WSL2 servicing a local replay socket. The takeaway is that runtime choice is workload-dependent, and the dominant penalty on the wrong runtime is jitter rather than drift.

Figure 5-Screenshot of the pipeline shutdown banner showing real Drift Human p50/p90/p99/p99.9 numbers on WSL against the replay server.

Async:

Figure 6-Screenshot of the pipeline shutdown banner showing real Drift Human p50/p90/p99/p99.9 numbers on Windows and This connects to the real Wikimedia SSE endpoint .

Async:

Figure 7-Tail-latency CDF plot generated by python scripts/plot_latency.py.

Figure 8-Tail-latency bar plot generated by python scripts/plot_latency.py.

Liu & Layland utilisation check. For two priority classes (Human, Bot) the schedulability bound is U ≤ 2(2^(1/2) − 1) ≈ 0.828. Observed CPU utilisation during a 60-second replay run was 0.03% average and 0.52% peak per logical core, sampled at 1 Hz from cumulative Get-Process CPU time via Windows Performance Counters on a 12-core machine. At an arrival rate of roughly 54 events per second, the pipeline consumes a negligible fraction of one core, far inside the 82.8% schedulability bound for two priority classes. The drift growth reported in §5.5 is therefore scheduler-latency-bound rather than utilisation-bound.

## Sync-Primitive Shootout

Measurement method: benches/sync_shootout.rs spawns 1–16 threads, each performing 1 000 increments (or push/pop cycles for DropOldestRing), and measures wall-clock time for all threads to complete. Criterion samples 50 iterations per (variant, thread-count) pair.

Figure 9-Criterion HTML report screenshot.

Figure 10-Sync-shootout throughput plot generated by python scripts/plot_shootout.py.

| Variant | 1t (µs) | 2t (µs) | 4t (µs) | 8t (µs) | 16t (µs) |
| --- | --- | --- | --- | --- | --- |
| std::sync::Mutex | 59.5 | 114.9 | 166.9 | 336.4 | 624.7 |
| parking_lot::Mutex | 70.3 | 101.9 | 160.1 | 281.6 | 515.0 |
| crossbeam_channel | 62.8 | 117.6 | 194.2 | 373.8 | 686.7 |
| DashMap | 80.5 | 128.3 | 198.0 | 335.6 | 602.2 |
| DropOldestRing | 87.7 | 124.7 | 225.2 | 395.5 | 744.1 |

At 16 threads, parking_lot::Mutexparking_lot is the fastest variant at 515.0 µs, about 21% faster than std::sync::Mutex at 624.7 µs (the original draft's 2× claim does not hold at the measured thread counts). This corresponds to a throughput of roughly 31.1 Mops/s for parking_lot against 25.6 Mops/s for std::sync. DashMap (602.2 µs) and crossbeam_channel (686.7 µs) sit between the two mutexes and below them respectively; DropOldestRing is the slowest at 744.1 µs, reflecting its mutex-protected VecDeque and dual-notifier overhead.

At 1 thread the ordering reverses. std::sync::Mutex is fastest at 59.5 µs, ahead of crossbeam_channel (62.8 µs), parking_lot::Mutexparking_lot (70.3 µs), DashMap (80.5 µs), and DropOldestRing (87.7 µs). parking_lot's short user-space spin pays off only once contention forces lock waiters to choose between spinning briefly and entering a kernel wait; at one thread there is no waiter and the spin is wasted overhead. The crossover between std::sync and parking_lot lands somewhere between 1 and 4 threads in this workload.

crossbeam_channel does not benefit from its message-passing abstraction on this workload because the bench is a single shared counter, and channel decoupling cannot reduce serialisation when every operation targets the same slot. DashMap's sharding helps less than expected: with one counter key, every operation hits the same shard and the data structure degenerates to a single mutex with hashing overhead. A fixed-slot atomic variant would likely outperform all five at high contention but only over a closed key set.

## Zero-Copy Heap Profile

Measurement method: The pipeline was built twice with --features dhat-heap: once with the default Cow<'a, str> parser and once with --features owned-baseline (which forces .to_string() on every field). Both ran for 60 seconds against the local replay server at 1× rate.

| Build | Total bytes alloc | Total blocks alloc | Peak bytes |
| --- | --- | --- | --- |
| Zero-copy (Cow) | 59 160 984 | 88 291 | 2 477 211 |
| Owned baseline | 71 787 424 | 250 009 | 2 493 388 |

The headline result is the allocation-event count: the owned baseline performs 250 009 allocations against 88 291 for the zero-copy build, a 2.83× ratio. The largest savings come from per-event String allocations for the user and server_name fields, which the zero-copy build borrows directly from the input buffer. Total allocated bytes drop only from 71.8 MB to 59.2 MB (about 18%) because most allocation volume is not in those parsed string fields, and peak resident bytes are essentially unchanged at 2.49 MB vs. 2.48 MB. The win here is allocator pressure and fragmentation cost, not working-set size.

The cow_was_borrowed counter measured 100% borrowed parses across the run with zero owned cases. Within this fixture, no user name or server name contained the JSON escape sequences that would force serde to materialise an owned String.

Figure 11-dhat web-viewer screenshot showing zero-copy vs. owned-baseline heap totals side by side(zero_copy).

Figure 12-dhat web-viewer screenshot showing zero-copy vs. owned-baseline heap totals side by side(owned_baseline).

## Fail-Safe Recovery

The tests/failsafe_recovery.rs integration test injects synthetic jitter by spin-looping in worker threads for 10 consecutive 1-second ticks (each tick records 1 000 samples at 5 ms jitter — well above the 1.5 ms enter threshold). The controller transitions to Degraded Mode after tick 10. The synthetic jitter is then removed; after 10 further ticks at 100 µs jitter (below the 0.8 ms recover threshold), the controller returns to Normal Mode.

The test confirms:

1. No false entry into Degraded Mode from a single high-jitter tick (requires 10 consecutive ticks).

2. No chattering: once in Degraded Mode, a single low-jitter tick does not trigger recovery.

3. Recovery timeline approximately 10 seconds, bounded by DWELL_TICKS.

In Degraded Mode, all bot: true events are dropped at the parser stage before enqueueing. Across the fixture this reduces queue depth by approximately 72% (2 211 of 3 078 events in the fixture are bot edits), giving human-priority events effectively exclusive access to the workers.

Figure 13- Demo-run terminal screenshot showing Degraded Mode ON — dropping Bot events followed later by Degraded Mode OFF — jitter recovered.

## Deadline Miss Analysis

A load-rate sweep was run on the local replay server under Windows, with both runtimes at 1×, 10×, and 100× replay rates for 30 seconds per run (six runs total). Across all six runs the pipeline recorded zero deadline misses and zero ring-buffer overflows. Even at 100× replay (sustained ~83 events/s for 30 s, n ≈ 156 000 events), neither runtime exceeded the 2 ms micro-deadline on any event. What changes under load is drift, not miss rate. Async p99 Human drift grows from 209 µs at 1× to 2 605 µs at 100×, a factor of 12.5; threaded p99 Human drift grows from 436 µs to 3 654 µs, a factor of 8.4. The threaded runtime's p99.9 Bot drift at 100× spikes to roughly 69.8 ms, an outlier consistent with kernel-scheduler tail behaviour rather than queue saturation. The fail-safe Bot-drop mechanism described in §3.5 was therefore never triggered in any of these runs; its protective effect is reserved for jitter-driven (rather than load-driven) degradation, as designed.

| Rate | Runtime | n_human | n_bot | p99 Human drift (µs) | p99 Bot drift (µs) | Deadline miss % | Overflow % |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1× | async | 970 | 750 | 209.0 | 406.0 | 0.00 | 0.00 |
| 1× | threaded | 971 | 751 | 435.7 | 665.6 | 0.00 | 0.00 |
| 10× | async | 8 714 | 7 245 | 300.5 | 580.1 | 0.00 | 0.00 |
| 10× | threaded | 8 714 | 7 245 | 621.6 | 978.9 | 0.00 | 0.00 |
| 100× | async | 84 875 | 71 267 | 2 605.1 | 5 050.4 | 0.00 | 0.00 |
| 100× | threaded | 84 642 | 70 934 | 3 653.6 | 18 006.0 | 0.00 | 0.00 |

## Platform Methodology Note

Benchmarks ran on two platforms, not one. The Windows live SSE results (§5.1, "Windows live SSE" rows) were collected on Windows 11 with PowerShell, executing rts-cli run-asyncrts-cli and rts-cli run-threadedrts-cli against the live Wikimedia endpoint at https://stream.wikimedia.org/v2/stream/recentchange for 60 seconds per run. The WSL2 replay results (§5.1, "WSL2 replay" rows) were collected under WSL2 Ubuntu, with rts-cli driving the local rts-replay server feeding a recorded 60-second NDJSON fixture, also at 60-second runs. The synchronisation-primitive shootout (§5.2) was run on Windows as a native target via cargo bench -p rts-bench --bench sync_shootoutrts-bench and its numbers should be read as Windows-native. Criterion's default warm-up and 50 samples per cell were used for the shootout.

# Conclusion and Future Work

A Rust implementation can meet soft real-time constraints for the Wikimedia SSE firehose, but the choice between Tokio async and std::thread is workload-dependent rather than absolute. Async wins on the Windows live SSE endpoint, where it delivers tighter p99 human drift (76.6 µs vs. 121.9 µs) and roughly 115× tighter p99 jitter (1.5 µs vs. 172.3 µs). Threaded wins on the WSL2 replay workload, where it produces lower p99 human drift (79.2 µs vs. 113.7 µs) and tighter jitter (1.7 µs vs. 25.7 µs). Drift differences across both platforms stay within roughly a factor of two; jitter is where runtime choice shows up most sharply. The zero-copy parser eliminates about 65% of allocation events (88 291 vs. 250 009 blocks) and 18% of total allocated bytes, with a borrowed rate of 100% on the measured fixture. Peak resident bytes are essentially unchanged because the dominant heap consumers lie outside the parsed string fields. The hysteresis-controlled Degraded Mode fail-safe prevents bot-volume spikes from crowding out human-priority events at the cost of discarding low-priority data, an acceptable trade for a soft real-time pipeline.

For future work, three directions stand out:

1. `simd-json` for SIMD-accelerated JSON parsing. This could reduce the mandatory serde_json::from_str cost from ~1.5 µs to ~0.3 µs per event, shrinking the dominant hot-path term.

2. OS-level thread priorities via the thread_priority crate (Windows THREAD_PRIORITY_TIME_CRITICAL, Linux SCHED_FIFO). This would let the threaded runtime enforce human-priority processing at the OS scheduler level, not only at the data-structure level.

3. Work-stealing across priority lanes. A Tokio LocalSet-based implementation where high-priority tasks are never stolen across worker threads could reduce cross-core synchronisation cost.

# References

Avizienis, A., Laprie, J.-C., Randell, B., & Landwehr, C. (2004). Basic concepts and taxonomy of dependable and secure computing. IEEE Transactions on Dependable and Secure Computing, 1(1), 11–33. https://doi.org/10.1109/TDSC.2004.2

Klabnik, S., & Nichols, C. (2023). The Rust programming language (2nd ed.). No Starch Press.

Liu, C. L., & Layland, J. W. (1973). Scheduling algorithms for multiprogramming in a hard-real-time environment. Journal of the ACM, 20(1), 46–61. https://doi.org/10.1145/321738.321745

Reeves, G. E. (1997, October). What really happened on Mars? Risks-Forum Digest. Computing Research Repository. https://catless.ncl.ac.uk/Risks/19/54

Schmitt, O. H. (1938). A thermionic trigger. Journal of Scientific Instruments, 15(1), 24–26. https://doi.org/10.1088/0950-7671/15/1/305

Sha, L., Rajkumar, R., & Lehoczky, J. P. (1990). Priority inheritance protocols: An approach to real-time synchronization. IEEE Transactions on Computers, 39(9), 1175–1185. https://doi.org/10.1109/12.57058

Tene, G. (2013). HdrHistogram: A high dynamic range histogram [Software]. https://hdrhistogram.org

Thompson, M., Farley, D., Barker, M., & Gee, P. (2011). Disruptor: High performance alternative to bounded queues for exchanging data between concurrent threads [Technical report]. LMAX Exchange. https://lmax-exchange.github.io/disruptor/disruptor.html

Tokio contributors. (2024). Tokio: An asynchronous Rust runtime (Version 1.43) [Software]. https://tokio.rs

Wikimedia Foundation. (2024). EventStreams — Wikimedia developer documentation. https://wikitech.wikimedia.org/wiki/Event_Platform/EventStreams

crossbeam contributors. (2024). crossbeam-utils: Utilities for concurrent programming (Version 0.8) [Software]. https://crates.io/crates/crossbeam-utils

parking_lot contributors. (2024). parking_lot: More compact and efficient implementations of the standard synchronisation primitives (Version 0.12) [Software]. https://crates.io/crates/parking_lot